SDM300 Series
Set the ASCII commands to echo characters locally and insert line feed.

<1/ followed by the command and the _ character.

Example to get the Eb/No of the carrier

<1/EBN0_

Following commands are most commonly used.

<1/REM

Must be used to set the modem in remote mode

<1/CBER_

Corrected Bit Error Rate

<1/EBN0_

Eb/N0

<1/MR_

Modulator Rate and Carrier (3/4_192.0)

<1/DR_

Demodulator Rate and Carrier Size

<1/RSL_

Receive Signal Level

<1/IBFS_

Interface Buffer Fill Level

<1/RF_ON_

Turn Carrier ON
<1/RF_OFF_

Turn Carrier OFF

<1/MOP_-21.5
Set Modem Power to -21.5

Fault Summary

<1/MFS

Modem Fault Summary – then use MS or DS to see the fault

Modem Status

<1/MS_

Lists the status of all modulator components

<1/DS_

Shows the status of the demod components

Use CTL plus ] to terminate the session
